# USER'S MANUAL

VisualTraffic v2.5

Traffic Assignment Software

for Microsoft® Excel

## VisualSoftware Co.

#### www.vtraffic.com

## OVERVIEW

### VisualTraffic

VisualTraffic is a traffic assignment program used within Microsoft Excel for Windows. The program methods are based on manual methods that help the user visualize the traffic network. VisualTraffic frees the user from designing spreadsheets while at the same time allowing for diversity.

The user first draws the roadway network. This network then becomes the interface for all forecast data. In this way, the focus is on the study network - not on rows and columns of formulas.

VisualTraffic does not assign traffic onto networks using modeling algorithms. Instead, the user selects the routes between Origin-Destination couplets and inputs the appropriate assignment percentages. This gives the user direct control over the assignment with a minimum amount of data regarding the study network.

### Getting Started with VisualTraffic

Steps to install and run VisualTraffic software.

- Copy files from diskette to a single directory in your drive. The VisualTraffic program file vt.xla, demonstration file vtdemo.xla and template files vttemp.xlt, and sigtemp.s10, and the Help file vt.hlp must be located in the same directory. The file names should not be changed.

- Start Excel 5.0 (or later)

- From Excel, open the file named vt.xla. The VisualTraffic menu will appear in Excel.

- Select Demonstration from the VisualTraffic menu to get an overview of the commands.

### Licensing VisualTraffic

VisualTraffic is distributed to our customers unlicensed. Your copy of VisualTraffic must be given a license number after it is installed on a computer. The program will then allow larger traffic networks based on the version you purchased. You must contact VisualSoftware Company to obtain this license number. Please refer to the About VisualTraffic section at the end of this manual.

### Program File vt.xla

The VisualTraffic program file vt.xla is an Excel add-in file. It is written in Microsoft Visual Basic for Applications (VBA). All VisualTraffic commands and procedures are contained within this file, with the exception of the Demonstration command. Procedures for the Demo. are contained in vtdemo.xla. Please refer to the Excel User's Guide for more information about Add-In files and the Tools-Add-In command.

### Demonstration File vtdemo.xla

The VisualTraffic demonstration file vtdemo.xla contains a set of procedures that will create a sample project. The traffic forecast data is input by the program into dialog boxes as the demonstration proceeds. The VisualTraffic program file vt.xla must be opened in Excel before the demonstration can be started.

### Workbook Template File vttemp.xlt

The VisualTraffic template file vttemp.xlt is a customized Excel workbook designed to be used with VT commands. It contains one Network sheet and the Data sheet. This file will be opened when the VT command New Workbook is selected. Since vttemp.xlt is a template file, Excel will automatically make a copy of it when it is opened, leaving the template unchanged. While in Excel, this resulting file may be saved as a workbook using any file name and directory.

### SIG/Cinema Template File sigtemp.s10

VisualTraffic can be used to export traffic volumes directly into SIG/Cinema files. Sigtemp.s10 is a SIG/Cinema file that is used by VisualTraffic as an initial file to export traffic volumes into. You may customize this template file in SIG/Cinema software and add any default values for convenience. Save the customized file using the same name and directory. This will be your default file when the command Export / SIG/Cinema is selected. SIG/Cinema is a signal optimization program from KLD Assoc., Inc. and Polytechnic University.

### User’s Manual file (MS Word) manual.doc

VisualTraffic User’s Manual in the MS Word v6.0 format. Also available in ASCII text file.

### User’s Manual file (ASCII) manual.txt

VisualTraffic User’s Manual in ASCII text file format. Also available in MS Word v6.0 format.

### On-line User’s Manual, WinHelp File vt.hlp

This help file is a Windows Help v3.1 file containing this User’s Manual. It can be reviewed using your Windows Help application or by selecting the Help command within the VisualTraffic menu while in Excel.

### Order Form File vtorder.xls

This Excel 5.0 workbook file contains an order form for VisualTraffic, along with an invoice for your records. Please use this order form when purchasing VisualTraffic software directly from VisualSoftware Co.

### Network Sheets

The vttemp.xlt file contains one copy of a Network sheet. Network sheets are the main interface for the user to represent, forecast, and output traffic assignments. Each VT workbook may include more than one, user-named, Network sheet. Network sheets contain Node circles and formatted cells for a traffic network. Approach and departure sum formulas are also in place at each Node.

Since some export commands use the Network sheet names as file names you should follow the stricter naming guidelines for file names. Therefore, Network sheet names should contain only uppercase and/or lowercase characters except the following characters: period (.), quotation mark ("), slash (/), backslash (\), brackets ([ ]), colon (:), semicolon (;), vertical bar (|), equal sign (=), comma (,), and space ( ).

The Node circles provide a reference for VisualTraffic when the user picks it. This is done by pressing the left mouse button while over the Node circle. The arrow will change into a hand symbol when it is placed over the Node circles. Node circles should remain in the original positions. Excel oval objects should not be added to any Network sheet, although other drawing object types may be used.

Nodes have ID numbers based on the position of the Node in the matrix. Node number one is always the top, left Node in the matrix. The Nodes are numbered sequentially from left to right and from top to bottom. Node numbers are used in Report and Export commands to identify the position of the data within the Network. Node numbers do not display on the Network.

The user determines how much data is represented on any Network sheet. For example; the first Network sheet may contain only existing roads and traffic volumes, while the second sheet may include just the forecast project trips to one or more Zones.

Network sheets may also be used to record distribution percentages, percentage of forecast traffic from a project, or routes and their percentages. These optional Networks are created by using basic VisualTraffic commands and may require modifying Trip, Distribution, or Assignment values temporarily.

### Data Sheet

The vttemp.xlt file contains the worksheet Data. The Data sheet stores distribution percentages, route descriptions and their assignment percentages. The Data sheet provides a tabular format to input and review information about the Networks. Only one Data sheet may be kept in each VisualTraffic workbook and must not be renamed.

Excel list filters have been included for convenience when working with many couplets or when multi-Trip Generator networks are used. Refer to the Excel User’s Guide for more information on filters.

Subtotal formulas are also in place at the top of the Distribution and Trip columns. These formulas calculate the total distribution or trip quantities that are visible due to any filtering of data.

The data on the sheet is maintained by VisualTraffic and should be edited using the appropriate VisualTraffic commands. The Distribution and Route percentages may be edited directly in the cells and are shown in blue.

## COMMAND SUMMARY

### New Workbook

Begins a new workbook designed for VisualTraffic commands which contains one Network sheet and the Data sheet.

### Network

A schematic representation of the study roadway with Zone and Trip Generator symbols.

#### Road

Draw (or delete) a road segment between two non-diagonal Nodes.

#### Label

Label or edit road names at a Node.

#### Refresh

Restore the appearance of the Roads, Zones, and Trip Generators as previously recorded.

#### Zoom Fit

Zoom-Fits selected area of Node matrix into screen.

#### Zoom Out

Zooms active sheet out at 10% increments.

### Study Node

A Node indicated as a Study Node by a solid red ring.

#### New

Draw ring at selected Node.

#### Delete

Delete ring at selected Node.

#### Volume

Input traffic volumes at an intersection. Also used to apply Growth factor to volumes.

### Zone

Graphic symbols representing areas of trip origin or destination with no trip generation.

#### New

Draw the Zone at the selected Node and input the Zone name.

#### Edit

Edit the Zone name. Changes on the Network sheet and the Data sheet.

#### Delete

Delete the Zone on the active Network sheet or throughout the workbook.

### Trip Generator

Graphic symbols representing areas of trip origin or destination with trip generation.

#### New

Draw the Trip Generator (TG) at the selected Node. Input the name and the Entering & Exiting Trip values.

#### Edit

Edit the TG name and Trip values. Changes on the Network sheet and Data sheets.

#### Delete

Delete the TG on the active Network sheet or throughout the workbook.

### Distribution

The percentage of total trips exiting from or entering a Trip Generator, associated with a Zone or another TG.

#### Setup

Prepares the Data sheet based on the Zones and TG’s present in the active workbook. Also used to update the Data sheet after Zones or TG’s are added to any Network sheet.

#### Edit

Edit the distribution percentages on the Data sheet. Distribution may be edited directly in cells on the Data sheet, shown in blue.

### Route

A continuous path over roadway segments beginning at a TG and ending at a Zone or another TG.

#### New

Stores a user-defined Route description on the appropriate row of Data sheet. When trips are sent along a Route, turning movement volumes will be posted only at the Nodes selected when Route was created. Nodes are shown in blue when Route is visible.

#### Review

Display selected Routes and edit Route usage values (approach & departure Routes shown separately). Values may be revised directly in cells on the Data sheet, shown in blue.

#### Delete

Display selected Routes and delete an individual Route (approach & departure Routes shown separately).

#### Show

Show selected Routes on the active Network sheet. Routes remain visible until ESCAPE key is pressed or Refresh command selected.

### Assignment

Post the trips onto a Network sheet.

#### Send Trips

Trips will be posted from the Data sheet onto the active Network sheet. The trips are based on the Trip Generators and Zones that are present on the active Network sheet, and the Routes that were previously created for those TG-Zone couplets.

#### Reassign

Sends trips along a user-defined path. Paths do not need to have endpoints at Zones or TG’s. Trip values sent are not associated with TG’s. Does not store path description or Trips on Data sheet.

#### Delete

Delete all turning movement volumes on the active Network sheet.

#### Combine

Combine the turning movement volumes from one or more Network sheets with those on the active Network sheet. Combine functions are; add, subtract, multiply, or divide.

### Report

Tabular reporting of traffic forecast quantities on Excel worksheets.

#### Vol. Sum.

Summarized Volume Report of all Study Nodes, turning movement volumes on active Network sheet.

#### Vol. Det.

Detailed Volume Report of a selected Study Node, also identifies source of forecast trips.

#### Dist. Table

Distribution Report of all Zones and TG’s on Data sheet.

#### Trip Table

Trip Table Report of all Zones and TG’s on Data sheet.

### Export

Writing data out from VisualTraffic workbooks to external files.

#### TMC

Creates an ASCII file of turning movement volumes at Study Nodes on active Network sheet, in the Turning Movement Count format. For importing traffic volumes into LOS software.

#### SIG/Cinema

Exports turning movement volumes, road names & header information into SIG/Cinema files.

#### DXF

Creates an ASCII file of the Roads, Study Node circles and all text on active Network sheet, in the Data Exchange Format. For import into CAD software and some LOS software.

### Utilities

Several procedures that can be used throughout the forecast process.

#### Select Vol.

Selects the cells that contain the turning movement volumes at individual Node/s or throughout the Network sheet. Use this utility to edit, format, delete, etc.

#### H/S Nodes

Hide or show the Node circles.

#### H/S AP, DP

Hide or show the approach and departure link volumes.

#### Page Setup

Modifies the Excel page setup based on input from user, indicating number of Nodes across and down desired on each page printed.

### Help

On-line User’s Manual in the Windows Help v3.1 format.

### Demonstration

Demonstrates VisualTraffic commands by creating a small sample project.

### About VisualTraffic

Licensing and contact information.

## COMMAND INSTRUCTIONS

### General

The typical method when using an Excel command is to first select the cell range, then select a command, but in VisualTraffic (VT) the opposite is true for the majority of commands. Select the VT command, then click on the Node to begin the process. VisualTraffic commands remain active until another VT command is selected or until the ESCAPE key is used.

Some Excel commands such as Cut & Paste are limited when VisualTraffic is loaded to avoid introducing errors to the Network sheets or Data sheet. Restart Excel without VisualTraffic to reset the Excel commands.

### New Workbook

This command starts by prompting the user for a Node matrix size ( # across & # down ), it opens the workbook template vttemp.xlt and formats the Network sheet for the indicated matrix size.

The Excel file summary window is displayed at the end of the command. This additional information is optional.

Use Excel commands to copy more Network sheets within the active VT workbook or to rename them (do not rename the Data sheet). Your current license for VT may limit the matrix size. Maximum matrix size is 1,000  ( 25 across x 40 down ).

### Network

#### Road

Roads are drawn by sequentially clicking on Node circles which are across from each other. If a Node is selected which is diagonal from the last Node selected, a new road segment will be started. (Diagonal road segments are not possible in the spreadsheet environment)

To erase a road segment, select the Nodes at the endpoints of the segment.

To end the Road command click on the VT tool button with the mouse. This tool button appears on a floating toolbar when the Road command is started. Clicking this tool button will record the current roadway onto the Network sheet. Roads are recorded in the form of Excel notes in the cell beneath each Node circle.

If you create new road segments but fail to record your session with the VT button then that section of roadway will be deleted when the Refresh command is used.

Some VT commands (such as Send Trips or Combine) process only those Nodes which have recorded Road, Zone or TG elements.

Roadways are “drawn” by changing the pattern color of the cells to red.

#### Label

This command provides a dialog box method to input or edit road names at the selected intersection. Pick the Node at the intersection and enter road names of the four approaches. The TAB key (or Shift+TAB) indexes the selection of the edit boxes. These labels may also be input directly into the proper cells on a Network sheet. Road names are used to identify intersections on Reports and in export files.

#### Refresh

Clears the pattern color for all cells on the active Network sheet, then restores the cell pattern colors for Roads, Zones, and Trip Generators as previously recorded.

#### Zoom Fit

Select command then pick two Nodes, the first Node at the top-left corner of an area, the second Node at the bottom-right corner. VisualTraffic will Zoom Fit the selected area into the screen. Similar to Excel’s Zoom-Fit command.

If you select wish to Zoom into one Node, just select the Node twice.

#### Zoom Out

Zooms the active sheet out at 10% increments. Use Excel’s Edit Repeat command to continue Zooming out.

### Study Node

#### New

Select this command then select the Node/s that you wish to make Study Nodes. Some VT commands, such as Report and Export, will process only data from Study Nodes. The Study Node is indicated with a solid red ring. These rings are visible in printouts and DXF files.

#### Delete

Selecting a Study Node with this command active will change the red ring back into a dotted black Node circle which will not be printed. Data at this Node will not be included on Reports or in some export files.

#### Volume

Click on a Node after selecting this command and VT will provide a dialog box for volume input at that Node. This is a quick way to input existing volumes or modifying volumes for growth, passer-by trips, etc.

This command may be used on grouped Network sheets as well. In that case, the volumes will be duplicated at the same Node location on each selected Network sheet. Refer to the Excel User’s Manual for more information about grouping multiple sheets together.

### Zone

#### New

Create a new Zone symbol. Select the command, click on a Node, then enter the Zone name in the dialog box.

When creating the same Zone on another Network sheet, it must be created at the same Node location. For example; traffic network AM Future W/out has a “Zone 3” located at Node #17. If another traffic network also requires “Zone 3”, it must be placed at Node #17 as well. This is because there is only one Data sheet per workbook and it stores distribution, routes, assignments and trips for all Network sheets within that workbook.

Zones are “drawn” by changing the pattern color of the cells to gray.

#### Edit

Edit the Zone name on the Network sheet. If the Data sheet has already been Setup the Zone name will be edited on the Data sheet as well.

#### Delete

Select a Node when this command is active and VT will request if the Zone is to be deleted throughout the workbook.

If the answer is; Yes, the Zone will be cleared from all Network sheets in the active workbook. All rows on the Data sheet containing the Zone will also be deleted along with the Distribution, Route descriptions and Assignment percentages stored on those rows.

If answered; No, the Zone symbol will be deleted only on the active Network sheet.

Zones are “deleted” by changing the cell pattern color to none, and deleting the Zone name from the cell.

When you wish to move an existing Zone symbol to another Node location, first delete the Zone throughout the active workbook, then recreate it at the desired Node location.

### Trip Generator

#### New

Create a new Trip Generator (TG) symbol. Select the command, click on a Node, then enter the TG name, Entering and Exiting Trips in the dialog.

Trip values are written into cells on the Network sheet and into formulas on the Data sheet. The formulas on the Data sheet multiply the Distribution percentage by the Entering or Exiting Trips.

TG’s are “drawn” by changing the pattern color of the cells to gray with a dark cyan border to distinguish them from Zones.

#### Edit

Select Edit then click on a TG. A dialog box is opened showing the selected TG name, and Trip values. Editing of these will change the name and Trip values on the Network sheet and also change names and formulas on the Data sheet for distributed trips.

#### Delete

Select a Node when this command is active and VT will request if the TG is to be deleted throughout the workbook.

If the answer is; Yes, the TG will be cleared from all Network sheets in the active workbook where present. All rows on the Data sheet containing the TG will also be deleted along with the Distribution, Route descriptions and Assignment percentages stored on those rows.

If answered; No, the TG symbol will be deleted only on the active Network sheet.

TG’s are “deleted” by changing the cell pattern color to none, changing the borders to none, and deleting the TG name and Trip values from the cells.

When you wish to move an existing TG symbol to another Node location, first delete the TG throughout the active workbook, then recreate it at the desired Node location.

### Distribution

#### Setup

Use this command after all of the Zones and TG’s have been created within the workbook.

The Setup command will inventory all the Zones and TG’s in the active workbook and create ordered rows of Origin-Destination couplets on the Data sheet. The command also creates the formulas that calculate the distributed trips.

The Setup command must be used again if additional Zones or TG’s are created. Doing this will add any new Zones or TG’s to the existing O-D couplet list. Do not delete a Zone or TG from every Network sheet without also deleting it from the Data sheet. If this happens, leaving an unused Zone or TG on the Data sheet, then the Setup command will cause errors in the ordered rows on the Data sheet. This can be resolved by recreating the Zone or TG on a Network sheet then Delete it throughout the workbook.

Care must be taken when working with multi-Trip Generator networks: Only the Exiting Trips will be accounted for on rows with TG-TG couplets. Exiting Trips from the Origin TG will account for the Entering Trips to the Destination TG.

#### Edit

Select two Nodes corresponding to a Trip Generator-Zone couplet, a dialog box is then displayed requesting approach and departure distribution. The Origin and Destination names of your selected couplet are shown in the dialog. The Origin is the first of the two Nodes selected.

Distribution may be input or reviewed in this graphic form with the Network sheet. For greater speed, the distribution can be changed directly in the cells on the Data sheet, shown in blue.

Using the list filters and subtotals for distribution and trips will help to account for trips, especially in a case of multi-TG Networks. You can take greater advantage of the Excel filter by choosing a naming convention for the Zones or TG’s and using the Custom filter option. Naming all Zones with a common first character such as “z” or “-”, etc., and using the wildcard function in Custom filters will allow you to display just the Project Exiting distribution to all Zones, for example.

### Route

#### New

Routes need to be created so that turning movements can be posted from the Data sheet to a Network sheet. Routes are continuous paths along the roadway, starting at a Trip Generator and ending at a Zone or another Trip Generator.

To start a new Route select the command, then pick the Node of the Trip Generator where the Route begins. Pick the Nodes along the road sequentially until the Route ends at a Zone or another TG. The Route may be described by picking all the Nodes along it, or it may be described by picking just the starting point, the corners along the road and the end point. When trips are later sent along a Route, the trip volumes will only appear at the Nodes that were selected while creating the Route.

As the Route is being created over a road, the road segment will be highlighted blue and each Node that is selected to describe the Route will also become blue.

Once the Route has been completed, push the VT button on the floating toolbar that appears at the beginning of the Route sequence. This will bring up a dialog to request the assignment. percentages for each direction of the Route (under the Origin-Destination couplet names). The Origin is the first Node selected to begin the Route.

Route descriptions for both the approach and departure Routes and their assignment percentages will then be stored on the appropriate rows in the Data sheet which match the couplets. The Route description is recorded, in the form of an Excel note, in the Route cell. The assignment percentage is written to this cell as a value and is shown in blue.

Additional Routes may be added one after another. When finished with the command, press the ESCAPE key after the VT button to record the last Route and end the command. This will Refresh the Network sheet.

If an error is made during the creation of a Route such as skipping a Node or missing a turn, use the ESCAPE key to cancel that Route.

In the case of a one-way Route, an assignment percentage of zero may be input for the opposing direction.

Each Route description is limited to 25-Nodes. Each Zone-TG couplet is limited to 25 Routes (one direction).

#### Review

After a Route has been created and stored on the Data sheet, it may be reviewed on the active Network sheet with the assignment percentage and Origin-Destination names displayed. Select the command, then choose which of three Route selection methods to use. See “Route Selection Method” below for further instructions.

The Route/s that you selected will be displayed sequentially. The destination of the Route is shown with a yellow Node. Each Route direction is shown separately.

The assignment percentage may be changed as the Route is displayed. Index to the next Route in the list by selecting the OK button. If you have modified the assignment percentage in the dialog box the percentage will be changed on Data sheet before the next Route in the list is displayed.

#### Delete

This command functions similar to the Review command but allows the deletion of any of the Routes as they are displayed. See “Route Selection Method” below. When the Delete button is selected, the cell on the Data sheet which stores that Route direction will be deleted, and the next Route in the selection will be displayed.

Only one Route direction is shown and deleted at a time, so care must be taken to delete the opposite Route direction as well. It is suggested to input an assignment of zero for an unneeded Route direction (as in the case of a one-way road), instead of deleting a single Route direction. Deletion of one Route direction will leave subsequent Route pairs mis-aligned in the columns, making the Route pairs less obvious. However, VisualTraffic makes no distinction in that case.

#### Show

Use this command to show the selected Routes over the active Network as a permanent record or temporarily for printing. Pressing the ESCAPE key will Refresh the Network. See “Route Selection Method” below.

#### [Route Selection Method]

Three Route selection methods are available when Reviewing, Deleting, or Showing Routes. If a selected Route goes between a Zone-TG or TG-TG couplet which is not present on the active Network sheet it will not be processed. This exclusion also occurs during the Send Trips command.

- Route : Select Route/s by entering the Route cell reference/s into the dialog box. You may select the cells directly on the Data sheet that store the Routes (under column headings; Route 1, to Route 25). Several cells may be selected at a time by Shift+Picking or Ctrl+Picking the cells. Once you have selected the Route cells of interest click on the OK button to proceed.

- Node : Select all Route/s which include a specific Node in their descriptions. The specific Node may be a Study Node, Zone or Trip Generator as well other Nodes. Click OK then click on a Node of interest to proceed.

- TM (Turning Movement) : Select Route/s which include a specific turning movement in their descriptions. Specific turning movements are indicated by first entering an abbreviation into the dialog box, i.e.: “NBT” to indicate Northbound-through, or “EBR” to indicate Eastbound-right turn. Press the OK button, then select the Node of the turning movement. Use this method to locate errors in Route descriptions or to audit your forecast assumptions.

### Assignment

#### Send Trips

Once you have input your traffic forecast assumptions for a particular scenario; Trips, Distribution, Routes and their assignments, the resulting Trips are calculated into turning movements and posted onto the active Network sheet with this command.

Only Trips associated with Trip Generators and Zones on the active Network sheet will be posted. Trips between Zone-TG or TG-TG couplets which are not present on the active Network sheet will not be posted to the active Network sheet.

VisualTraffic will inventory the Zone-TG and TG-TG couplets present on the active Network sheet as valid couplets. VT then gets the calculated distributed Trip value from the fourth column in Data sheet for each valid couplet. It then multiplies this distributed Trip value by the assignment percentage of a Route and sends those Trips along that Route. For each valid O-D couplet, all Routes are processed sequentially from left to right. Then each valid couplet is processed from top to bottom of the Data sheet.

When the Trips are sent, the values will be added to values present in the turning movement cells from previous runs or from existing counts. Use the Assignment / Delete command before sending Trips in the case of a revised assignment.

If text exists in a cell intended for turning movement volumes a run-time error will occur. To find the offending cell/s use the following procedure: VisualTraffic command Network / Select Vol. [throughout], then use Excel command Edit / Go To / Special / Constants - Text. This will select all turning movement cells which have text in them.

#### Reassign

Use this command to post traffic volumes onto the active Network sheet along a traced out path. The path is created along road segments in the same manner as in the Route-New command. The path does not need to start or end at a TG or Zone. The input volumes for both approach and departure directions will be posted at the Nodes selected when the path was created, except at the path end points. A minimum of three Nodes are needed to describe the path. This command does not record the path or volume to the Data sheet.

Use this command to adjust traffic volumes on a Network due to Passer-By trips, Diverted trips or Reassignments around congested areas.

It may be useful to keep these manual volume adjustments on a separate Network sheet from other Trips which are posted from the Data sheet.

#### Delete

Selecting this command will delete all values in the turning movement cells on the active Network sheet. Use this before posting a revised assignment from the Data sheet to avoid double counting.

#### Combine

This command combines one or more Network sheet/s with the active Network sheet using arithmetic arguments. The dialog displays all Network sheets currently opened. These Network sheets may be in other opened VT workbooks.

Select the Combine function of either add, subtract, multiply or divide, then select the Network sheet/s to be combined by highlighting them in the list. Use Ctrl+Pick to select more than one Network sheet.

The turning movement volumes (TMV) on the selected Network sheet/s will then be combined with those on the active Network sheet. For example; [active Network TMV= active Network TMV (÷) selected Network TMV]

In the case of multiply or divide, only the first Network sheet selected in the dialog will be combined with the active Network sheet.

### Report

#### Vol. Sum.

Summarized Volume Report of all Study Nodes on selected Network sheets. Adds a new worksheet to the active workbook. Report tabulates turning movement volumes and link approach and departure volumes for each of the selected Network sheets.

Excel multiple sheet selection is allowed and is processed from left to right within the active workbook. Refer to the Excel User’s Manual for further instructions on selecting multiple sheets.

#### Vol. Det.

Detailed Volume Report of all Study Nodes on selected Network sheets. Adds a new worksheet to the active workbook. Report tabulates Origin-Destination source of forecast trips, and the resulting turning movement volumes at each Study Node.

Each valid O-D couplet will be reported. VisualTraffic will inventory all O-D couplets present on the active Network sheet, as in the Assignment / Send Trips command. Only trips associated with those couplets will be reported.

If a Study Node has traffic volumes other than forecast volumes from the Data sheet, then those volumes will be reported as being from Unknown Source. This would be the case for example, if the Assignment / Combine command was previously used.

Excel multiple sheet selection is allowed and is processed from left to right within the active workbook. Refer to the Excel User’s Manual for further instructions on selecting multiple sheets.

#### Dist. Table

Distribution Report tabulates all of the Distribution between Origin-Destination couplets. The report uses Zone and Trip Generator names from Data sheet. Adds a new worksheet to the active workbook

#### Trip Table

Trip Table Report tabulates all of the forecast Trips between Origin-Destination couplets. The report uses Zone and Trip Generator names from Data sheet. Adds a new worksheet to the active workbook

### Export

#### TMC

Creates an ASCII file of all Study Nodes on the active Network sheet, in the Turning Movement Count format. For importing traffic volumes to LOS software. Requests the Start time and number of 15-minute periods. Enter the Start time in military format, i.e.; 800, 1200, or 1315.

When using this command to import traffic volumes into Highway Capacity Software (HCS), you will need to change the Peak Hour Factor within HCS that is calculated from the TMC file. Note that VisualTraffic exports the total forecast volumes on the first 15-minute row, leaving subsequent rows equal to zero. This will give you a calculated PHF of 0.25 for a one-hour study period.

Names for the TMC files will be generated by VT using the following method: “TMC” for the first three characters, followed by the Node ID number. The extension is “VOL”. A file will be created for each Study Node on the active Network sheet and stored in the same directory as the active workbook.

#### SIG/Cinema

Turning movement volumes may be exported directly out to files for SIG/Cinema software. A default SIG/Cinema file is supplied with your VisualTraffic software; sigtemp.s10

When this command is selected, VT will look for a SIG/Cinema file that may have already been created. If no matching file is found, VT copies the file sigtemp.s10 into the directory of the active workbook file, then renames it. If, on the other hand, the file already exists it will be revised with the current turning movement volumes and header information.

One SIG/Cinema file will be created for each Study Node on each of the selected Network sheets.

Names for the SIG/Cinema files will be generated by VT from the first five characters of the active Network sheet name, followed by the Node ID number, then the extension “S10”. These files will be stored in the same directory as the active workbook.

The SIG/Cinema default file; sigtemp.s10 may be customized using SIG/Cinema software. Default values may be added to the supplied file to provide initial values for all subsequent files created from it.

#### DXF

Creates an ASCII file of the Roads, Study Node circles and all text on active Network sheet, in the Data Exchange Format. For import into CAD software and some LOS software.

A Dialog box requests the Scale Factor for the DXF. Each column and row in the Network smheet will be equal to one dimensional unit multiplied by the Scale Factor (default=100).

Names for the DXF files will be generated by VT from the first eight characters of the active Network sheet name, followed by the extension “DXF”. These files will be stored in the same directory as the active workbook.

### Utilities

#### Select Vol.

This command selects the turning movement volume cells at the selected Node/s or throughout the Network sheet. Use this to delete unneeded volumes or to format cells. You can select single, multiple or all Nodes with this command.

There may be forecast traffic volumes at non-Study Nodes that you wish to delete on finished Networks since these intermediate volumes may not represent total traffic volumes at those locations.

#### H/S Nodes

Hide or show Node circles. Most VT commands will not function when Node circles are hidden. You may want to hide Nodes before giving the workbook to non-users to avoid introducing errors in the workbook. Also, as a protective measure before archiving your project workbooks.

#### H/S AP, DP

Hide or show the rows and columns in a Network sheet that have the approach and departure link volumes. Road names are also on these rows and columns.

Use Hide to get a more compact Network sheet, showing just the Turning Movement Volumes.

#### Page Setup

This command modifies the Excel page setup for printing. Select the Network sheet/s that you wish to setup for printing, select this command, then indicate the desired number of Nodes across and down per printed page.

VT will calculate the print area so that each automatic page break will occur in the middle between Nodes. VT also calculates the number of pages across and down in order to print the entire Node matrix. VT will also set the Zoom value for the page and set manual page breaks on the Network sheet. VT  will then determine Portrait or Landscape page orientation. The page setup will be applied to all selected sheets.

If you wish to use the Scaling Adjust option, to take advantage of the manual page breaks, you must select it in the Excel Page Setup command. The Zoom value will be the calculated value from the VT Page Setup command.

### Help

On-line User’s Manual in the Windows Help v3.1 format. This on-line manual may be reviewed by opening the file vt.hlp from within your Windows Help application or by selecting this command from the VT menu.

### Demonstration

This command creates a small sample project. Progress through the demonstration by answering the dialog boxes. The file vtdemo.xla must be present in the same directory with the VisualTraffic program file vt.xla.

### About VisualTraffic

Licensing and contact information. When registering your VisualTraffic copy with VisualSoftware Co. the software must be running on the computer that you intend to keep it on. Telephone VisualSoftware Co. when you are ready to register the license number.

Selecting the Register License button on the About VisualTraffic window will prompt you for a specific License number that will match the Key number given in the lower left portion of the window. Give this Key number when calling and you will be given the matching License number.